Fluvial / Pluvial Modelling Workflow

# 0. Overview

This guide covers the scripts used in the fluvial and pluvial modelling workflow. Most are adapted from the Canada flood project; some are new. The aim is to simplify and unify the process, from input preparation through running the model to distribution, and to keep inputs easy to track.

The scripts run in pixi, the preferred environment. Reasons for using it:

- Portability: the same environment runs on every machine.

- Maintainability: libraries and GDAL are configured once, with no manual installs.

- Easy updates: the environment is refreshed with a single command.

Initial setup is minimal: pixi is installed once, then scripts are run from their folder.

The QGIS plugin gives an overview of the data for a given area. Its main purpose is to show which inputs exist. Data can be filtered by AreaID.

The database is filled automatically by the individual tasks in the main extraction. A new input is saved to the database when created and should then be visible there.

# 1. One-time setup

## Pixi

Pixi is installed once per machine. On Windows, open PowerShell and run:

powershell -ExecutionPolicy Bypass -c "irm -useb https://pixi.sh/install.ps1 | iex"

This downloads the latest version, installs it to %UserProfile%\.pixi\bin, and adds that folder to PATH. Restart the terminal afterwards so the pixi command is available. Update later with pixi self-update.

## QGIS plugin

The plugin, European flood data catalogue, is installed from the internal plugin repository. It bundles all required libraries, so no extra installation is needed.

## Database access

No client software or manual configuration is required. Access uses an aonid and a password, distributed separately. The scripts and the plugin connect with these credentials.

# 2. Running scripts

The workflow uses a single pixi environment, defined by one pixi.toml. Input extraction lives in 01_Extract_FLPL; domain creation and the TUFLOW workflow live in 02_run_simulation.

From the folder that contains pixi.toml, set up the environment once:

pixi install

Then run a script with:

pixi run python script_name.py

pixi install reads pixi.toml and builds the environment. It is run once, and again only after the environment changes. pixi run python script_name.py runs a script inside that environment.

# 3. Workflow

The scripts below are run in order; each entry states what the script does and what to set in it. Return periods and their parameters are defined in the multi_scenario CSV file in 03_bat, and the multi_scenario setting selects which file is used.

## Input extraction

main_extract.py runs the input extraction tasks for the area of interest. New inputs it creates are written to the database. A few steps use separate standalone scripts, marked below.

- MERIT segments: selects the river network (MERIT) segments for the area of interest and adds new segments upstream.

- pluvial_grid_preselect: creates a preselection of the pluvial domains. Edit it manually and save it to the edited file; that file is used to build the mask that clips the virtual pipes.

- vector_data_extract: from predefined layers, creates the city selection to process further, and the empty files to fill in later: culverts and ZSH polygons (common), RN, WB and tunnels (pluvial), channels and levees (fluvial).

- dtm_extract: creates the extracted DTM for editing and a high resolution DTM for pluvial.

- A01_add_dtm_db.py (standalone, not part of main_extract): once the DTM has been edited, registers the current file in the database.

- waterbodies_raster: creates the waterbodies raster from the edited waterbodies layer, later used to lower the DTM. Run it only after the waterbodies layer has been edited manually.

- buildings_raster: creates the buildings raster.

- pluvial_dtm_recalculate: recalculates the DTM using the waterbodies and buildings rasters.

- virtual_pipes: creates the 1d_pits file from roads, industrial estates, Manning values and parking.

- zsh_culverts: creates the ZSH output for culverts.

- zsh_channels: creates the ZSH output for channels.

- zsh_levees: creates the ZSH output for levees.

- A02_create_bcdbase.py (standalone, not part of main_extract): copies the discharge values into the bc_dbase structure on the R: drive. Run it before domain creation.

## Fluvial

- FL01_get_inputs_from_db.py: reads the area inputs (DTM, Manning, ZSH culvert, channel and levee lines, building and waterbody rasters) from the database and writes them into a setup YAML for the area. Set AREA_ID and DTM_RESOLUTIONS, run it, then open the generated YAML and fill any entry left blank. Before running 01_tu_extract_run.py, set main_setup_yaml in tu_what_to_do.yaml to the path of the generated file.

- 01_tu_extract_run.py: builds the model inputs and the TUFLOW text files for each domain, and writes one YAML per domain. Settings are in tu_what_to_do.yaml: set area, the Q_Do_ switches, and multi_scenario. Make sure main_setup_yaml points to the setup file created by FL01_get_inputs_from_db.py.

- FL03_tuflow_test_run.py: runs a short local CPU test for each domain to confirm it starts. Set DOMAINS_TO_DO, RESOLUTION, TUFLOW_EXE, and the folder paths.

- FL04_copy_to_distribution.py: checks the test run finished, copies each domain to the distribution machine, and writes the distribution YAML. Set DATA_STORAGE, DOMAINS_TO_DO, RESOLUTION, and the paths.

## Pluvial

- S01_define_domains.py: creates the empty domains (clip, code, rainfall and outlet boundary layers). Set DOMAINS_TO_DO and OUT_FOLDER_DOMAINS.

- S02_reference_domains.py: builds the reference Excel listing the domains and their DTM resolutions. Set GRID, OUT, SELECTED_DOMAINS, and CITY_NAME.

- PL01_get_inputs_from_db.py: reads the area inputs (DTM, soil, Manning, ZSH lines, buildings and waterbodies, virtual pipes) from the database and writes them into a setup YAML for the area (setup_<AREA>_<dtm>m_<mesh>m.yaml in the _yaml folder). Set AREA_ID, DTM_RESOLUTION, and MESH_RESOLUTION, run it, then open the generated YAML and fill any entry left blank. Before running PL02_run_extract_domains.py, set main_setup_yaml in tu_what_to_do_city_yaml.yaml to the path of the generated file.

- PL02_run_extract_domains.py: builds the model inputs and the TUFLOW text files for each domain, and writes one YAML per domain. Settings are in tu_what_to_do_city_yaml.yaml: set the area and domains, the Q_Do_ switches, multi_scenario, multiscenario_test, and is_test_run. Make sure main_setup_yaml points to the setup file created by PL01_get_inputs_from_db.py.

- PL03_tuflow_test_run.py: runs a short local CPU test for each domain. Set DOMAINS_TO_DO, RESOLUTION, TUFLOW_EXE, and the paths.

- PL04_copy_to_distribution.py: checks the test run, copies each domain to the distribution machine, and writes the distribution YAML. Set DATA_STORAGE, DOMAINS_TO_DO, RESOLUTION, and the paths.